Map Area Tool

Complete Documentation

Version: 1.0

Purpose: Precise area and perimeter measurement for rooftops and regions from map screenshots

# 1. Executive Summary

The Map Area Tool is a desktop application designed to accurately measure areas and perimeters of structures and regions using Google Maps screenshots. The tool eliminates manual estimation by providing precise calculations based on pixel-to-metric calibration and the Shoelace polygon area formula.

## Key Features

- Scale Calibration: Two methods for establishing pixel-to-real-world distance mapping

- Polygon Drawing: Interactive pixel-level precision for drawing measurement boundaries

- Multi-Unit Support: Automatic conversion between m², ft², yd², km², and acres

- Export Options: Save measurements in JSON, CSV, or TXT formats

- Real-Time Visualization: Instant area, perimeter, and bounding box calculations

- Zoom & Pan: Full image manipulation for precise measurement

# 2. System Overview

## Purpose

This tool solves the problem of manual area estimation from map screenshots by providing:

- Accurate measurement using mathematical polygon area calculations

- Scale-aware calculations through calibration against map scale bars

- Multiple unit support for different measurement contexts

- Data export for integration with other systems

- Intuitive UI for minimal learning curve

## Supported Image Formats

PNG, JPG, JPEG, BMP, TIF, TIFF, WebP

## Operating Environment

| Property | Value |
| --- | --- |
| Platform | Windows, Linux, macOS |
| Python Version | 3.7+ |
| UI Framework | PyQt5 |
| Image Processing | Pillow (PIL) |

# 3. Architecture

## High-Level Design

┌─────────────────────────────────────────────┐

│         Main Application Window             │

│  (app/window.py - MainWindow class)         │

├─────────────────────────────────────────────┤

│  ┌──────────────┐  ┌──────────────────────┐ │

│  │  UI Sidebar  │  │   Canvas (Drawing    │ │

│  │  (Panels)    │  │   & Visualization)   │ │

│  │              │  │   (CanvasWidget)     │ │

│  │ • Tools      │  │                      │ │

│  │ • Scale Cal. │  │  • Polygon points    │ │

│  │ • Results    │  │  • Ruler line        │ │

│  │ • Export     │  │  • Zoom/Pan          │ │

│  └──────────────┘  └──────────────────────┘ │

└─────────────────────────────────────────────┘

↓

┌─────────────────┐

│  Math Engine    │

│ (geometry.py)   │

└─────────────────┘

↓

• Polygon area (Shoelace formula)

• Perimeter calculation

• Bounding box computation

• Unit conversions

## Module Organization

| Module | Class | Responsibility |
| --- | --- | --- |
| window.py | MainWindow | Application orchestration, signal routing, results calculation |
| canvas.py | CanvasWidget | Image rendering, user interaction, polygon/ruler drawing |
| geometry.py | (Functions) | Mathematical calculations and unit conversions |
| panels.py | ScalePanel, ResultsPanel, ToolPanel | UI components for sidebar |

# 4. Installation & Setup

## Prerequisites

- Python 3.7 or higher

- pip package manager

## Step 1: Install Dependencies

pip install -r requirements.txt

## Step 2: Verify Installation

python main.py

The application window should open successfully.

## Dependencies Reference

PyQt5>=5.15.0    # GUI framework

Pillow>=9.0.0    # Image processing

# 5. User Guide

## Starting the Application

### Option 1: Open with File Dialog

python main.py

### Option 2: Open Specific Image Directly

python main.py /path/to/screenshot.png

## Core Workflow

The measurement workflow consists of four steps:

### Step 1: Load Image

- Click 📂 Open Image [Ctrl+O] button

- Navigate to and select a map screenshot

- Image will be loaded and centered on canvas

Important: Ensure the image includes:

- The Google Maps scale bar (bottom-left corner)

- The complete area/structure to measure

- No overlapping elements

### Step 2: Calibrate the Scale

Scale calibration is mandatory for accurate real-world measurements. Without it, measurements display in pixels only.

Method A: Ruler Tool (Recommended)

- Press R (or click ↔ Measure Scale Bar)

- Mode indicator changes to "Mode: Measure Scale Bar" (green)

- Click the left endpoint of the Google Maps scale bar

- Click the right endpoint of the scale bar

- System captures pixel distance

- In the Scale Calibration panel:

  - Measured: shows the pixel distance (e.g., "150.3 px")

  - Update the "Real dist:" field to match the scale bar label (e.g., 50 for "50 m")

  - Click ✓ Apply (ruler method)

- Status indicator changes to "CALIBRATED ✓" with the calculation

Tips for Accuracy:

- Zoom in on the scale bar before measuring (scroll to zoom)

- Ensure you measure the exact endpoints, not slightly beyond

- Most Google Maps scale bars show 50m or 100m

- For best results, use rulers on the smallest zoom level of the map

Method B: Direct Entry

If you already know the pixel-to-meter ratio:

- In the Scale Calibration panel, locate "px / m:"

- Enter the known conversion factor (e.g., 6.67 for 6.67 pixels per meter)

- Click ✓ Apply (manual entry)

### Step 3: Draw the Polygon

- Press P (or click ⬡ Draw Polygon)

- Mode indicator changes to "Mode: Draw Polygon" (green)

- Click around the boundary of the area to measure

- Close the polygon using one of three methods:

  - Press Enter

  - Double-click

  - Click near the first point (within snap radius) to close via snap

Constraints:

- Minimum 3 points required to form a polygon

- Maximum number of points: unlimited

- Polygon must be closed to calculate area

### Step 4: Review Results & Export

Once a closed polygon is created with scale calibration:

Results Panel Display

| Metric | Description |
| --- | --- |
| Area | Total enclosed area in selected unit (highlighted in green) |
| Bound. W | Bounding box width |
| Bound. H | Bounding box height |
| Perim. | Total perimeter of the polygon |
| Points | Number of vertices in the polygon |
| Scale | Current calibration (px/m) |

Unit Selection

Use the "Unit:" dropdown in the Measurements panel to switch between:

- m² — Square meters (default)

- ft² — Square feet (US standard)

- yd² — Square yards

- km² — Square kilometers

- acre — Acres

Length measurements automatically convert to corresponding units:

| Area Unit | Length Unit |
| --- | --- |
| m² | meters (m) |
| ft² | feet (ft) |
| yd² | yards (yd) |
| km² | kilometers (km) |
| acre | meters (m) |

Export Results

- Click ↗ Export Results [Ctrl+E]

- Choose save location and filename

- Select format:

  - JSON — Structured data with metadata

  - CSV — Comma-separated values (spreadsheet compatible)

  - TXT — Plain text with key-value pairs

## Canvas Controls

| Control | Action | Purpose |
| --- | --- | --- |
| Scroll | Zoom in/out | Magnify or reduce view |
| Middle Mouse Drag | Pan canvas | Move visible area |
| Left Click | Place/measure point | Draw polygon or ruler |
| Double Click | Close polygon | Complete measurement shape |

## Keyboard Shortcuts

| Key | Action | Mode |
| --- | --- | --- |
| P | Activate polygon mode | Any |
| R | Activate ruler mode | Any |
| Enter | Close polygon | Drawing polygon |
| Ctrl+Z | Undo last point | Drawing or measuring |
| Esc | Clear all points | Any |
| Scroll | Zoom | Any |
| Middle drag | Pan | Any |
| Ctrl+O | Open image dialog | Any |
| Ctrl+E | Export results | Any |

## Performance Constraints

- Maximum Image Size: 16,384 × 16,384 px (GPU dependent)

- Maximum Points: 1,000+ per polygon (CPU dependent)

- Typical Operations: <100ms

- Zoom Levels: 0.1× to 10.0× typical range

# 6. Pixel-to-Real-World Mapping

## Calibration Concept

The core problem: Image pixels have no inherent physical dimension. To measure real-world areas, we must establish a scale factor that converts pixel distances to metric distances.

## Scale Factor Definition

Scale factor (ppm) = pixels per meter

Example: If the scale bar shows "50 m" and spans 333 pixels, then:

ppm = 333 pixels / 50 meters = 6.66 pixels/meter

## Calibration Methods

### Method 1: Ruler Tool (Recommended)

Process:

- User identifies scale bar in map screenshot

d_pixels

d_meters

ppm = d_pixels / d_meters

Advantages:

- Direct measurement from image

- No external data needed

- Self-verifying (user sees the measurement)

Sources of Error:

- Imprecise endpoint clicks

- Scale bar ambiguity

- Pixelated endpoint artifacts

### Method 2: Direct Entry

Process:

- User provides known conversion factor

ppm = user_input

Use Cases:

- Repeated measurements on identical zoom levels

- Measurements from pre-calibrated screenshots

- Known map projections

## Pixel Coordinate System

All polygon points are stored in image coordinate space:

- (0, 0) = Top-left corner of image

- x increases rightward

- y increases downward

- Units: pixels (integer or float)

## Distance Conversion Formula

Distance_meters = Distance_pixels / ppm

Distance_unit   = Distance_meters × conversion_factor

Distance_meters = 250 / 6.66 = 37.54 meters

Distance_feet = 37.54 × 3.28084 = 123.2 feet

# 7. Mathematical Algorithms

## Polygon Area: Shoelace Formula

The Shoelace formula (also called the surveyor's formula or Gauss area formula) computes the area of a polygon from its vertex coordinates.

### Formula

For a polygon with n vertices (x₀,y₀), (x₁,y₁), ..., (xₙ₋₁,yₙ₋₁):

A = (1/2) | Σ (x_i * y_(i+1) - x_(i+1) * y_i) |    for i = 0 to n-1

where indices wrap around: (x_n, y_n) = (x_0, y_0)

### Implementation

def polygon_area_pixels(points: List[Tuple[float, float]]) -> float:

"""Shoelace formula. Returns area in px²."""

n = len(points)

if n < 3:

return 0.0

area = 0.0

for i in range(n):

j = (i + 1) % n

area += points[i][0] * points[j][1]

area -= points[j][0] * points[i][1]

return abs(area) / 2.0

### Properties

- Complexity: O(n) — linear in number of vertices

- Vertex Order Invariant: Works for clockwise or counter-clockwise ordering

- Precision: Limited by floating-point arithmetic (IEEE 754 double precision)

- Range: Valid for any simple polygon (no self-intersections)

### Example Calculation

For a rectangle with corners (0,0), (100,0), (100,50), (0,50):

A = (1/2)|0·0 - 100·0 + 100·50 - 100·0 + 100·50 - 0·50 + 0·0 - 0·50|

A = (1/2)|5000 + 5000| = 5000 px²

(Correct: 100 × 50 = 5000 px²)

## Perimeter Calculation

The perimeter is the sum of Euclidean distances between consecutive vertices:

P = Σ d(p_i, p_(i+1))   for i = 0 to n-1

where d(p_i, p_(i+1)) = sqrt((x_(i+1)-x_i)² + (y_(i+1)-y_i)²)

### Implementation

def perimeter_pixels(points: List[Tuple[float, float]]) -> float:

"""Total perimeter of a closed polygon in px."""

n = len(points)

if n < 2:

return 0.0

total = 0.0

for i in range(n):

j = (i + 1) % n

dx = points[j][0] - points[i][0]

dy = points[j][1] - points[i][1]

total += math.sqrt(dx * dx + dy * dy)

return total

## Bounding Box Calculation

Returns the axis-aligned bounding rectangle (x_min, y_min, width, height):

def bounding_box_pixels(points: List[Tuple[float, float]]) -> Tuple[float, float, float, float]:

"""Returns (min_x, min_y, width_px, height_px)."""

xs = [p[0] for p in points]

ys = [p[1] for p in points]

return min(xs), min(ys), max(xs) - min(xs), max(ys) - min(ys)

## Area Unit Conversions

### Conversion Factors

1 m²   = 1.0 m²

1 m²   = 10.7639 ft²

1 m²   = 1.19599 yd²

1 m²   = 1e-6 km²

1 m²   = 0.000247105 acres

### Algorithm

### Implementation

def area_px_to_unit(area_px2: float, scale_px_per_m: float, unit: str) -> float:

area_m2 = area_px2 / (scale_px_per_m ** 2)

conversions = {

"m²":   1.0,

"ft²":  10.7639,

"yd²":  1.19599,

"km²":  1e-6,

"acre": 0.000247105,

}

return area_m2 * conversions.get(unit, 1.0)

### Length Unit Conversions

| Area Unit | Length Unit | Conversion |
| --- | --- | --- |
| m² | m | 1.0 |
| ft² | ft | 3.28084 |
| yd² | yd | 1.09361 |
| km² | km | 0.001 |
| acre | m | 1.0 |

# 8. API Reference

## MainWindow Class

Location: app/window.py

### Constructor

MainWindow(image_path: Optional[str] = None)

Parameters:

- image_path — Path to initial image file (optional)

### Public Methods

| Method | Signature | Purpose |
| --- | --- | --- |
| _open_dialog() | () | Display file open dialog |
| _load_image() | (path: str) | Load image from file path |
| _set_mode() | (mode: str) | Switch between "polygon" and "ruler" modes |
| _refresh_results() | () | Recalculate and display results |
| _export_results() | () | Export measurements to file |

## CanvasWidget Class

Location: app/canvas.py

### Signals

polygon_changed = pyqtSignal(list)   # emits list of (x, y) tuples

ruler_measured  = pyqtSignal(float)  # emits pixel distance

status_changed  = pyqtSignal(str)    # emits coordinate info string

### Public Methods

| Method | Signature | Purpose |
| --- | --- | --- |
| load_image() | (path: str) → bool | Load and display image |
| set_mode() | (mode: Mode) | Switch drawing mode |
| clear() | () | Clear all points |
| undo_last() | () | Remove last point |
| close_polygon() | () | Finalize polygon |
| get_polygon_points() | () → List[Tuple[float, float]] | Get current points |

### Properties

@property

def mode(self) -> Mode:

"""Current drawing mode (POLYGON, RULER, or PAN)"""

## Geometry Module

Location: app/geometry.py

### Area & Perimeter Functions

def polygon_area_pixels(points: List[Point]) -> float

"""Compute polygon area using Shoelace formula. Returns area in px²."""

def perimeter_pixels(points: List[Point]) -> float

"""Compute total perimeter in pixels."""

def bounding_box_pixels(points: List[Point]) -> Tuple[float, float, float, float]

"""Return (min_x, min_y, width, height)."""

def distance_pixels(p1: Point, p2: Point) -> float

"""Euclidean distance between two points in pixels."""

### Unit Conversion Functions

def area_px_to_unit(area_px2: float, scale_px_per_m: float, unit: str) -> float

"""Convert pixel area to specified unit (m², ft², yd², km², acre)."""

def length_px_to_unit(length_px: float, scale_px_per_m: float, unit: str) -> float

"""Convert pixel length to appropriate unit based on area unit."""

def length_unit_label(unit: str) -> str

"""Return length unit abbreviation corresponding to area unit."""

def pixels_to_meters(px: float, scale_px_per_m: float) -> float

"""Direct pixel-to-meter conversion."""

## Panel Classes

Location: app/panels.py

### ScalePanel

class ScalePanel(QWidget):

scale_changed = pyqtSignal(float)  # emits ppm value

def set_measured_pixels(px: float) -> None

"""Update display after ruler measurement."""

def get_scale() -> float

"""Return current scale factor (px/m)."""

### ResultsPanel

class ResultsPanel(QWidget):

unit_changed = pyqtSignal(str)  # emits selected unit

def update_results(data: dict) -> None

"""Update displayed measurement results."""

@property

def unit(self) -> str

"""Return currently selected unit."""

### ToolPanel

class ToolPanel(QWidget):

sig_open = pyqtSignal()

sig_polygon = pyqtSignal()

sig_ruler = pyqtSignal()

sig_close = pyqtSignal()

sig_undo = pyqtSignal()

sig_clear = pyqtSignal()

sig_export = pyqtSignal()

def set_active(mode: str) -> None

"""Highlight active tool button."""

# 10. Export Formats

## JSON Format

File Extension: .json

### Structure

{

"timestamp": "2026-05-09T16:30:48.762167",

"unit": "ft²",

"scale_px_per_m": 6.662655774577884,

"points_count": 4,

"polygon_points_px": [

[509.06, 97.7],

[885.2, 104.87],

[863.68, 732.11],

[484.46, 720.84]

],

"area": "57305.7963 ft²",

"perimeter": "988.2053 ft",

"bounding_width": "197.3318 ft",

"bounding_height": "312.3999 ft"

}

### Field Descriptions

| Field | Type | Description |
| --- | --- | --- |
| timestamp | ISO 8601 | Export time with microsecond precision |
| unit | string | Selected unit (m², ft², yd², km², acre) |
| scale_px_per_m | float | Calibration factor (pixels per meter) |
| points_count | integer | Number of polygon vertices |
| polygon_points_px | array | Polygon vertices in image coordinates |
| area | string | Total area with unit suffix |
| perimeter | string | Polygon perimeter with unit suffix |
| bounding_width | string | Bounding box width with unit suffix |
| bounding_height | string | Bounding box height with unit suffix |

## CSV Format

File Extension: .csv

### Structure

timestamp,2026-05-09T16:30:48.762167

unit,ft²

scale_px_per_m,6.662655774577884

points_count,4

polygon_points_px,"[[509.06, 97.7], [885.2, 104.87], [863.68, 732.11], [484.46, 720.84]]"

area,57305.7963 ft²

perimeter,988.2053 ft

bounding_width,197.3318 ft

bounding_height,312.3999 ft

Format Details: RFC 4180 compliant, two columns (key/value), importable into spreadsheet software.

## Text Format

File Extension: .txt

### Structure

timestamp: 2026-05-09T16:30:48.762167

unit: ft²

scale_px_per_m: 6.662655774577884

points_count: 4

polygon_points_px: [[509.06, 97.7], [885.2, 104.87], [863.68, 732.11], [484.46, 720.84]]

area: 57305.7963 ft²

perimeter: 988.2053 ft

bounding_width: 197.3318 ft

bounding_height: 312.3999 ft

Format Details: Plain text key: value format, one measurement per line, human-readable.

# 11. Troubleshooting & Accuracy

## Common Issues

### Issue: "NOT CALIBRATED" Status

Symptoms: Area shows in pixels (e.g., "5,280 px²") instead of real units

Cause: Scale factor not established or not applied

Solution:

- Press R to activate ruler mode

- Click both ends of the Google Maps scale bar

- Verify pixel measurement appears in Measured: field

- Enter the scale bar distance in Real dist: field

- Click ✓ Apply (ruler method)

Verification: Status should change to green "CALIBRATED ✓", area should display with unit.

### Issue: Polygon Not Closing

Symptoms: Polygon remains open after drawing points; Enter key has no effect

Cause: Fewer than 3 points drawn, or polygon already closed

Solution:

- Verify at least 3 points are visible on canvas

- Verify polygon is not already closed (check mode indicator)

- Press Enter or double-click

- Press Esc to clear all and start over

### Issue: Measurements Appear Incorrect

Symptoms: Area calculations seem too large/small compared to visual estimate

Causes:

- Imprecise scale calibration — Most common

- Wrong scale bar measured — Multiple scale indicators on complex maps

- Imprecise polygon drawing — Polygon doesn't follow boundary closely

- Map distortion — High latitude regions distorted in Web Mercator projection

### Issue: Image Won't Load

Symptoms: Error dialog "Could not load image" appears

Solution:

- Verify file exists and path is correct

- Try different image format (convert to PNG if needed)

- Check file permissions (should be readable)

Supported formats: PNG, JPG, JPEG, BMP, TIF, TIFF, WebP

### Issue: Zoom/Pan Not Working

Solution:

Ctrl+O

- Try alternative pan: middle mouse button + drag

- Click on canvas to ensure focus

- On trackpad: use two-finger scroll for zoom

## Accuracy Improvements

### 1. Scale Bar Measurement

- Zoom to 100% pixel ratio (1:1)

- Use scale bars at smallest zoom level (least distortion)

- Click endpoints very carefully, aligned with scale bar edges

- Measure multiple times (at least 2) and average if consistent

Expected precision: ±0.5-1%

### 2. Polygon Drawing

- Zoom to see boundary details (scroll to zoom in)

- Place vertices every 5-20 pixels along boundary

- For curved edges, use even more vertices (every 2-5 pixels)

- For rooftops: trace entire perimeter, not just visible corners

Expected precision: ±2-3%

### 3. Map Selection

- Use map tiles from same zoom level (consistent scale throughout)

- Avoid screenshots at extreme latitudes (>60°) where distortion is high

- Verify scale bar is consistently labeled across entire image

Expected precision: ±1-2% at equator, ±5-10% at 60° latitude

### 4. Mathematical Precision Limits

| Factor | Limit | Notes |
| --- | --- | --- |
| Pixel grid | ±0.5 px per point | Discrete sampling |
| Floating-point | ~15 significant digits | IEEE 754 double precision |
| Shoelace formula | ±0.01% area error | Accumulated rounding |
| Map projection | ±1-5% at high latitude | Web Mercator distortion |

### 5. Validation Workflow

- Pre-measurement: Select high-quality screenshot, verify scale bar is readable, confirm entire area is visible

- Calibration: Measure scale bar 3 times, calculate average ppm, manually verify measured_px ÷ ppm ≈ labeled_distance

- Drawing: Start with simple shapes to validate setup, add many vertices for actual shape

- Verification: Compare area to visual estimate, confirm perimeter is reasonable

- Documentation: Export results to JSON with timestamp, note calibration method and zoom level

## Performance Considerations

| Operation | Target | Maximum Acceptable |
| --- | --- | --- |
| Image load | <1s | <5s |
| Polygon calculation | <100ms | <500ms |
| Zoom operation | <50ms | <200ms |
| Export | <1s | <5s |

If performance degrades:

- Reduce polygon complexity — Fewer vertices per shape

- Use smaller images — Scale down screenshots before loading

- Restart application — Clear memory cache

- Check system resources — Other programs consuming memory/CPU

## Combined Error Budget

Total error² = Calibration_error² + Drawing_error² + Projection_error²

Example (1000 m² area):

- Calibration: ±1% = ±10 m²

- Drawing:     ±3% = ±30 m²

- Projection:  ±2% = ±20 m²

Combined (RSS): sqrt(10² + 30² + 20²) = ±37 m² ≈ ±3.7%

Result: 1000 ± 37 m²

# Appendix A: Glossary

| Term | Definition |
| --- | --- |
| Shoelace Formula | Mathematical algorithm for computing polygon area from vertex coordinates |
| ppm | Pixels per meter; the scale calibration factor |
| Web Mercator | Map projection used by Google Maps; introduces distortion at high latitudes |
| Polygon | Closed shape formed by connecting sequence of points |
| Bounding Box | Smallest axis-aligned rectangle containing entire polygon |
| Snap Radius | Distance threshold (10px) for closing polygon by clicking near first point |
| Perimeter | Total length of polygon boundary |
| IEEE 754 | Standard for floating-point arithmetic; ~15 decimal digit precision |
| RFC 4180 | Standard for CSV file format |
| ISO 8601 | Standard for date/time representation (e.g., 2026-05-09T16:30:48) |

# Appendix B: Mathematical References

## Unit Conversion References

### Length Conversions

- 1 meter (m) = 3.28084 feet (ft)

- 1 meter = 1.09361 yards (yd)

- 1 kilometer (km) = 0.621371 miles (mi)

### Area Conversions

- 1 m² = 10.7639 ft²

- 1 m² = 1.19599 yd²

- 1 m² = 0.000247105 acres

- 1 hectare = 10,000 m² = 2.471 acres

- 1 acre = 43,560 ft² = 4,046.86 m²

## Web Mercator Distortion

At latitude φ, the scale factor is:

k(φ) = sec(φ) = 1 / cos(φ)

Example scale factors:

- At equator (φ=0°): k=1.000 (no distortion)

- At 30° latitude: k=1.155 (15.5% stretch)

- At 45° latitude: k=1.414 (41.4% stretch)

- At 60° latitude: k=2.000 (100% stretch)

This distortion affects only the north-south direction; east-west is preserved.